ASSIGNMENT 5

Q1. What is the differnce b/w

a. Tie and Strut

b. Column and Strut

c. Short Column and Long Column

d. Buckling Load and Crushing Load

Q2.  Define Slenderness Ratio and give classification of Columns.

Q3.  What are different end condition for a column and give there respective equivalent length.

Q4. What are the assumptions made in Euler's Theory and discuss the Limitations.

Q5. A circular bar of length 4m and 60mm dia. is used as a column with both ends fixed. determine buckilng load for the column using Euler Formula. Given (E = 2x105N/mm2)

Q6. a rectangular column of length 6m is fixed at both ends.If section of column is 100mm wide and 200  mm. find euler buckling load for the column

Q7. A solid circular bar of a 4 meter length and 50mm dia was found to be extended 4.6 mm under tensile load of 50 kn calculate the euler buckling load for the column if both ends fixed .

Q8. A cast iron column of hollow circular section external dia 250mm and thickness of 25mm as to transmit axial compressive laod p .the length is 7 meter with both end hinged take f.o.s = 8 determine p . if rankine conatants : sigma c = 560 and r =1/600

Q9. A hollow cylindrical c.i column is 6 meter and both ends fixed.calculate internal minimum dia meter of column if it has to carry a safe load of 300 kn. f.o.s = 4 take internal dia meter as 0.7 times external dia meter .. if rankine conatants : sigma c = 550 and r =1/600