ASSIGNMENT - 6

Q1.  Discuss th limits of eccentricity in following cases

1.  Rectangular   2. Circular   3. Hollow Circular  Section

Q2.  A short column of solid rectangular section 150*80mm carries a vertical load of 140 k.n whose line of thirst is on the central line parallel  to longer  side and distant 30mm from centre line parllar to short side . dtermine the maximum and minimum intensity of stresses

Q3. A solid column of circular section of 800mm in dia carries an eccentric load of 230 k.n . find the greatest intensity which the laod can have with producing tension in the cross section

Q4.  A short column having a ext dia 300mm thickness of metal 40mm carries a load of 400 k.n . the line of action of which coincides with axis of column  it also carries a second laod of 200k.n ,whose line of action is 80mm from the centre draw the stresss distribution diagram at the base of section

Q5. A concrete bloack of radius 1.2 meter weight 100k.n is subjected to an eccentricity of laod if maximum stress is 2.5 times the mean stress

Q6. A hollow rectangular column having external dimensions 60mm wide 40mm thick and internal demsensions 20*20 mm. calculate the safe load that can be placed at an eccentricity of 12 mm on a plane bisecting thickness if the maximum stress not to exceed 30 n/mm 2

Q7. A cylindrical masonry column is 1.5 meter in dia it is acting upon by a horizontal wind presure of 1200 n/mm 2 calculate the height through which chimney can built without causing tension at the base take the unit weight of maasonry as 22 k.n /m3

Q8. A square chimeney 16 m high has an opening 1.5*1.5 meter and wall thickness of 0.3 m draw stress distribution diagram at the base of section if horizontal presure is 900 n/m2 . take specific weight of the masonry as 22 k.n /m3

Q9. Differntiate between concentric and ecccentric laod on a column.

Q10. What is middle third rule . Explain