Surveying II

Assignment Ist

- Define

- Contour	b) Contouring	c) Contour Interval	d) Horizontal Equivalent

- Write Characteristics of Contour.

- Write Direct and indirect method of Contouring.

- Explain temporary and permanent adjustment of Transit Vernier Theodolite.

Assignment IInd

- What are Curves and give classification of Curves

- a) Write down elements of Simple Curve

- What is designation of a Curve

- A Simple Circular Curve have radius of 573 m the tangents intersect at chainage 1060m and the angle of intersection is 120°. Find tangent distance, length of Long Chord, chainage at beginning and end of Curve, Degree of Curve and No. of Full Chord and Sub Chord.

Assignment IIIrd

- a) Define Tacheometry. What are the instruments used in it and write down the Characteristics of Tacheometry.

b) Write the theory of Stadia Tacheometry

2. The constants (f/i) and (f+d) of a certain tacheometer were 100 and 0.3 respectively. The readings of the three Diaphragm hairs on a staff held at distant object were found to be 1.16, 2.40, 3.44 the telescope being horizontal. Find the horizontal distance of the staff from instrument axis and R.L of the staff point. If the R.L of the Instrument Axis is 80m.

3. Two distances of 20m and 100 m were accurately measured and the intercept on the staff between the outer stadia webs were 0.19m at the former distance and 0.99m at the later distance. Calculate the Tacheometric Constant

4. Derive Equation for Fixed hair method of tacheometry when the line of sight is inclined.