Name: - Vaibhav Pathania

Trade: - Arch. Asstt.

Roll no. : - 282/2017

6th Sem

Worksheet- 2

Questions: -

Write short notes on following with sketches: -

- Conceptual /Study model

- Plaster of Paris (POP)

- Various type of furniture shown in a model

- Railing design

- Equipment used in model making

- Materials used for parking lot in a model

- Commonly used scales for making different types of model

- Throw some light on the past, present and future of model making

- Vehicles, vegetation and human figure forms an important role in architecture. Explain how?

- Materials used to represent roads, paving, trees and grass in a model.

NOTE:-

The answer should be of 6-10 lines with sketches

Answers: -

- Conceptual models: - It is a representation of a model, made of the composition of concepts which are used to help people know, understand, or simulate a subject the model represents. It is also a set of concepts. Some models have physical objects; for example, a school miniature model which is constructed, and shows the concept the used by maker to show how it will look.

Study models: - These are like presentation models are often used in client meetings or in colleges to convey a finished design in miniature while study models are used by architects as part of the design process. Study models are the equivalent of a three-dimensional sketch and allow us to explore and design ideas quickly.

- Plaster of paris, quick-setting gypsum plaster consisting of a fine white powder, which hardens when moistened and allowed to dry. Known since ancient times, plaster of paris is so called because of its preparation from the abundant gypsum found near Paris.

It is used in houses nowadays for making false ceilings, pop designs on ceiling and for plasters in(ortho) sector. It is also extensively used in models too for making hills, rivers and many more.

- Various types of furniture are used and shown in a miniature or architecture models as , Architects has to show interior as well as exterior of the model if we are making any sectional model we have to show interior as well as exterior so we use or construct miniature tables, chairs, human figures, sofas etc. and for showing exterior we use or construct the trees, grass, street furniture like: - benches, street lights, post boxes, phone booths etc.

- Railing Design: -

- Equipments Used for model making are: -

- Craft Knife and Cutting mat : - These are essential model making tools. They can be used to cut model making foam boards as well as paper and card.

- Craft Knife and Steel Rular : - A craft knife should always be used with a steel ruler when cutting straight lines, ensuring that hands and fingers are kept behind the cutting edge.

- Glue: - It is used for pasting the cut pieces and used for pasting building blocks too.

- Scissors: - It is also used for cutting paper, cellophane sheet, cloth, mats etc.

- Handsaw: - It is used to cut hard material like PVC pieces, wooden pieces and mostly used to cut base of the material.

- Sandpaper: - It is used to grade the cut pieces of wood and PVC to give it a smooth and straight finish.

Thus these types of equipments are use for making model.

- Materials used for making Parking Lot: - For making parking lot in model we can use black paper or grey paper for showing road of the parking lot or we can use black or grey colour saw dust for the road texture for making partitions for the parking we can use white pen of 0.7- 0.10 grade tip for marking on paper or we can use white coloured mount board pieces or we can use white and yellow tapes etc.

- The most widely used scales for building models are 1:100, 1:150, 1:200 and 1:300. The balconies, windows and detailed building facades, etc can be shown at these scales. 1:100 to 1:300 scales are widely used for making building models. In these scale, the building facade can be well represented.

- Throw some light on the past, present and future of model making

- Vehicles, vegetation and human figure forms an important role in architecture as we know in architecture model we represent the models in very detailed form and helps the people to know easily what we are showing so we use Vehicles to show roads or parkings etc., Vegetation to show the landscaping in model, human figures to make it like realistic.

- Materials used to represent roads, paving, trees and grass in a model are: -

To make Roads we use: -

Black or grey chart paper and we can also use black or grey saw dust for giving texture to road.

To make Paving we use: -

Cardboard, mount board and 0.6- 0.8 mm PVC etc. with various thickness according to the scale of model.

To make Grass we use: -

We can use artificial grass mats, green mats, green coloured saw dust, velvet sheet, cloth etc. for making grass in a model.